Braille Sense Display driver - Guide for Installation the JAWS


Installation Guide for Braille Sense Display Driver for JAWS
2011-01-26

HIMS International Corporation.
Please be sure that you are running Jaws 10 or higher as this driver is compatible only with JAWS 10 and above. Earlier versions of JAWS 10 may not work correctly.

Please be sure you have installed Jaws version 10 or above on your PC before proceeding with this installation. This driver can be used with the following Sense Notetakers:
1. Braille Sense Classic v5.2 or higher.

2. Braille Sense PLUS B32 v5.2 or higher.

3. Braille Sense OnHand v5.5 or higher.

4. Braille Sense PLUS Qwerty v5.2 or higher.

5. Voice Sense v5.2 or higher(only with Keyboard).

6. Voice Sense Qwerty v5.2 or higher(only with Keyboard).

· Some languages do not support the Voice Sense for the terminal for the Screen Reader.

You may obtain the JAWS driver for the Braille Sense or Voice Sense using Braille Keyboard and Qwerty Keyboard from the links below:
Driver Installation: 
http://www.himsintl.com/upload/JAWS/HIMS_Braille_Driver_For_Jaws_V2_1.zip.
You may obtain documents containing default key descriptions for the Braille Sense or Voice Sense using Braille Keyboard and Qwerty Keyboard from the links below:
Please note: If you are using a Qwerty keyboard, there are some key limitations so please check the following documents carefully.
1. Braille Keyboard: 

MS Word document: www.himsintl.com/upload/JAWS/Hot-Key_Define_Jaws_Braille.doc.
TXT document: www.himsintl.com/upload/JAWS/Hot-Key_Define_Jaws_Braille.txt.
2. Qwerty Keyboard:

MS Word document: www.himsintl.com/upload/JAWS/Hot-Key_Define_Jaws_QWERTY.doc.
TXT document: www.himsintl.com/upload/JAWS/Hot-Key_Define_Jaws_QWERTY.txt.
This document consists of guides for installation/deletion, USB connection, Serial Connection and Bluetooth connection.

· HOW TO INSTALL Braille Sense Display Driver.
1. Locate the driver installation file on your PC and Run it; the installation wizard will be started as below.

A. To start installation, click the “Install” button at the bottom of the screen.
[image: image1.png]rer for JAWS 1.0 =101 ]|

Welcome to the Braille Sense Driver
for JAWS 1.0 Setup Wizard

This wizard wil quide you throuch the instalatin of raile
Sense Driver for JAVIS 1.0,

Itis recommended that you dose al other appications
before starting Setup. Tris il make it possble o update
relevant system fles without having to reboot your
computer.

Clck Instal tostart the nstalaton

el ]| conce





B. At this time, if Jaws is not installed on your computer, the installer will warn you that “JAWS is not installed” and you are not able to continue with the installation.

2. Once the installer completely installs the necessary files for using the Braille Sense with JAWS, the “Device Driver Installation Wizard” will appear as below. This will install the driver for connecting between Braille Sense and JAWS via USB cable.

A. To start installation, click the “next” button.
[image: image2.png]D

evice Driver Installation Wizard

=2

Welcome to the Device Driver
Installation Wizare

This wizard helps you nsal the scftware divers that some
computers devices need n order to work

To cortinue. dick Next

Cancel

=
{1





B. Select “HIMS USB” in the list box and click the “finish” button
[image: image3.png]D

=2

evice Driver Installation Wizard

Completing the Device Driver
Installation Wizard

The diivers were successfuly nstaled on this computer

You can now connect your devics o tis computer.  your device:
came with nstructions, please read the st

Driver Name. Status
Microsoft (imsusb) Sam... Readyto use.

Frish Cancel





C. The installation process is complete.

D. To close the  installation wizzard, click the “finish” button.

[image: image4.png]raille Sense Driver for JAWS 1.0 =101 ]|

Completing the Braille Sense Driver
for JAWS 1.0 Setup Wizard

Braile Sense Driver for JAVIS 1.0 has been instalied on your
computer.

Clck Finsh to cose ths wizard.

Erish





· HOW TO CONNECT Braille Sense as a USB Braille display.
1. When you have installed the Braille Sense Display driver for JAWS, Connect the USB OTG cable to the USB OTG port on the Braille Sense and the wider end of the cable should be connected to the USB port on your computer.
2. Run “Terminal for the Screen Reader” under the “Utilities” menu on the Braille Sense.
3. Select “USB port (u)” and the “New Hardware Wizard” should appear on your PC.
[image: image5.png]Found New Hardware Wizard

=2

Welcome to the Found New
Hardware Wizard

Windows wil search for curent and updated software by
looking on your computer, on the hardware installation CD., or on
the Windows Update Web sie (with your pemission).

Head oir privacy polisy

Can Windows comnectto
software?

indows Update to search for
€ Yes, tis time only

€ Yes. now and everytime | connect 2 device.
€ No.not his tme:

Cick Nextto contine.

Cancel





4. Choose “Yes, this time only” and then select the “Next” button.
[image: image6.png]Found New Hardware Wizard

Welcome to the Found New
@ Hardware Wizard

Windows will search for cument and updated software by
ookt onyourcompue: on e haee moalion . or n
the Windows Uipdate Web site (with your pemission).
Readarpmcroier

Can Windows comnectto
software?

indows Updete o search for
@ Nes. this e oniy:

€ Yes. now and everytime | connect a device.
€ No.not his tme:

Cick Nextto contine.

Cancel

=
g





5. Select “Install the software automatically” and then select the “Next” button.
[image: image7.png]Found New Hardware Wizard

“This wizard helps you nsal software for

HIMS USE DRIVER

(). ¥ your hardware came with an installation CD
<& or floppy disk. insert it now.

Vihat do you wart the wizard to do?

@ psaithe soivars audoraicaly Fesommanded)
€ Istlfrom st orspecic ocation (Advancee)

Cick Nextto contine.

Cancel

=
g





6. On the next screen, select the “Next” button.
[image: image8.png]Found New Hardware Wizard

Please wait while the wizard searches...

HINS USB DRIVER

o> || cones





7. Select the ‘finish’ button to complete the installation.

A. In the future, when you connect the Braille Sense to your PC, there is no need to install or register the USB driver.

[image: image9.png]izard

Found New Hardware

=2

Completing the Found New
Hardware Wizard

“The wizard has finished instalng the software for

mg U

Cick Fish to close the wizard

Fiish





8. Run JAWS and select “Braille” under the “Options” menu.
9. Select “Braille Sense” as the “Default Braille Display” and “USB” as the port.

A. That’s all that is necessary to connect the Braille Sense to JAWS via USB cable.
[image: image10.png]The Active Brailedisplay s Mo Display.

-
——

Mority Setings.

[ Show Eraille Load Error

Translaton Table:

US_ricode B

‘Ackd Braile Display.

advanced.

Cancel

[Ditermines current braille display being used





· HOW TO USE a Serial Connection.
1. Connect the Braille Sense to your PC using the serial cable that is included in the Braille Sense package.
A. Voice Sense and Braille Sense OnHand do not have a Serial port so you can’t connect these devices via serial connection.
2. Run “Terminal for the Screen Reader” under the “Utilities” menu.
3. Select “Serial port (s)” to use the serial interface.
4. Please make sure that the message “terminal mode” appears on the Braille display.
5. In JAWS, select “Braille Sense” as the “Default Braille Display” and the physical COM port(most likely COM1) as a port.

A. That’s all that is required to connect the Braille Sense to JAWS via Serial cable.
· HOW TO CONNECT through Bluetooth Serial
1. Click the “Add” button in the Bluetooth Dialog in XP SP2.
[image: image11.png]Bluetooth Devices

Devices | Optons | COM Pors | Hardware

add. ][ Remoe | [ Propeties

[ eoow




2. Check the box that reads, “My device is set up and ready to be found” in order for another Bluetooth device to find your PC.

[image: image12.png]Welcome to the Add Bluetooth
Device Wizard

Belore proceeding efe 1o the “Blustooth” section ofthe
device documertaiion. Then set up your devics 5o that your
computer can fnd it

Tunitan
Make it discoverable (visible]

Give it 3 name [optional)

Press the bulton an the battam of the device
keyboards and mice orip)

1 devics i sef U and ready 1 B found

@ Add orly Bluetosth





3. Your computer will begin searching the Bluetooth devices nearby your PC.

[image: image13.png]Add Bluetooth Device Wizard

Select the Bluetooth device that you want to add.

) Ifyou dont see the device that you want 1o add, make sure tht tis
tuned on. Follow the setup instructons that carie with the device,
and then olick Search Again.





4. Select the “Braille Sense PLUS” as the device you want to connect to.
[image: image14.png]Add Bluetooth Device Wizard x|
‘Select the Biuetooth device that you want to add. e;“’
Razer Orochi =
Aready connected
BookSenseXT g vmee
B89 vew device B89 e device

Aople Wieless Keyboard MRCH
Aready connected New device |

i Khun Jana2| MacBook Ar

i) Hyou dont sesthe devioetht you want o add, make sur et s
fumed on_ Folow the seup insuctionsthal came wih the GeWce.  Search Agan
andhen clck Search Agai

I v





5. Enter a 4-digit PINCODE of your own choosing to pair between the two devices and then click the “next” button.
A. At this time, the Braille Sense will ask for the PINCODE you just entered on your PC.
[image: image15.png]Add Bluetooth Device Wizard

Do you need a passkey to add your device?

To answer this queston, refe t the "Blustoot” secton of the documentation that came with
your device. I the documertation speciliss 2 passkey, use that ore.

O Chaose a passkey for me.
O Use the passkey found in the documentatior:

(@ Let me choos my own passkey:

O Dorttuse & passkey

Yo should ahways use & passkey, uless your device doss ot suppot one. We
tecommend using a passkey tha is 8 a 16 digtslong. The longe the passkey, the
more secue it vl be.





B. Pairing between two devices.
[image: image16.png]Add Bluetooth Device Wizard

Windows is exchanging passkeys.

‘When instructed below, enter the passkey using you Bluetoolh device.

For more information about entering a passkey, sez the documentation that came with your
device

+ Cannecting.

¥ Please enter the passkey on your Bluetooth device now.

Passkey: 1234

Instaling Blustooth device.





C. Complete pairing between two devices.
[image: image17.png]Add Bluetooth Device Wizard

Windows is exchanging passkeys.

‘When instructed below, enter the passkey using you Bluetoolh device.

For more information about entering a passkey, sez the documentation that came with your
device

+ Cannecting.

 Please enter the passkey on your Blustooth device now.

Passkey: 1234

» Installing Bluetooth device.





D. Your PC will create the virtual Bluetooth serial port automatically.
[image: image18.png]Completing the Add Bluetooth
Device Wizard





E. Please make sure that the Bluetooth serial port is configured correctly.(outgoing port: COM17) –  the port number may be different from the example per each PC.
[image: image19.png]etooth Devices

Devioss | Opons COM Pots | Harare |

‘This computeris using the COM (sera)pors listed below. To
determine whether you need a COM por ead the documertation
that came with your Bluetooth device.

For. Diection_[ Name.
COMT7  Outgoing  BralleSensePLUS Bue Tooth Seral
COMIE  Incoming  BraleSensePLUS

Add Herove

Leam more about Bustooth COM ports.

ok | ool | s |





6. Run “Terminal for the Screen Reader” under the “Utilities” menu.
7. Select “Bluetooth serial port (s)” to use the serial interface.
8. Please make sure that the message “terminal mode” appears on the Braille display.
9. Run JAWS and select “Braille” under the “Options” menu.
10. Select “Braille Sense” as the “Default Braille Display” and “COM17” as the output port.
A. That’s all that you must do in order to connect the Braille Sense to JAWS via the Bluetooth Serial Port.
[image: image20.png]The Active Brailedisplay s Mo Display. Translaton Table:

[ralesense -]

Seftinge:
Com7,

[ oty Seffings. ]

[ Show Eraille Load Error

‘Ackd Braile Display.

advanced.

o

Cancel

[Adjust communication settings for current braille display.





[image: image21.png]t options for the Braille Sense braille display.

Port nformation

Selectthe output port
[cont
Selectthe cutput port speciications [COM2
cons
cons
cons
conts
conr
cona i
Press Insert + F1 for Hel coue





· HOW TO REMOVE the Braille Sense Display driver
1. Navigate to the JAWS installation folder.

A. For example, “C:\Program Files\Freedom Scientific”.

2. Navigate to “\Shared\Drivers\Braille\1.0\ThirdParty\brlsense” from the folder.

3. Run “Uninstaller.exe”
4. If JAWS is currently running, the following message will be displayed.

A. ‘In order to complete the uninstallation action, you must reboot the computer. If you do not want to reboot the computer now, the uninstallation will be completed the next time you reboot the computer.’
5. If JAWS is not currently running, the folder will be deleted automatically.

If you have any questions, please call Tech Support at +82-42-864-4601(ext. 100) or email supports@himsintl.com. 

HIMS International Corporation.

139-9, Gajung-dong, Yuseong-gu, Daejeon, KOREA, 305-350 
Phone: +82-42-864-4601(ext. 100)
www.himsintl.com
HIMS International Corporation.


