Braille Sense Plus Firmware download manual

- What stuffs you need

- Windows PC (XP or 7) and USB cable

- Memory card (SD, CF, USB)

- H332B_USB download.zip file

- Braille Sense USB driver.exe

- JFlash folder: rwnkdub.exe

- image folder: nk_140212.bin

  - Download nkdub.bin

    - Connect Braille Sense to PC via Serial cable.

    - Run DNW.exe(in H332B_USB_Download folder) and set properties of serial port.( DNW: It is a terminal program to receive information from Braille Sense via Serial cable)

<DNW main window>

    - In case of using COM1, set property as below picture.

<Set property of serial port >

    - Click “Connect” menu to receive information from Braille Sense.

< Connect Serial Port>

    - Run the Braille_SENSE_USB_Driver.exe.(for downloading nkdub.bin via USB cable.)

    - Connect Braille Sense to PC via USB cable(the cable was given with Braille Sense package).

    - Input ‘U’ in the boot menu of DNW window.

- You well see the next screen in order to install USB driver

- Select ‘Yes, this time only’ and then press ‘Next’ button.

- Select ‘Install the software automatically’ and then press ‘Next’ button

- You will see the next screen and then press ‘Next’ button.

- Press ‘Continue Anyway’ button

- Press ‘finish’ button then installation will be finished

- You can see next screen after installation was finished. It means Braille Sense is ready to download the data by USB cable.

    - Run rwnkdub.exe(in Jflash folder).

- Next screen shows downloading progress.

- Next screen shows the upgrading progress.

- You can see the boot menu in the DNW window if the upgrading is finished successfully.

  - Update to Braille Sense

    - Copy nk_140212.bin(main image in image folder) to memory card. (CF or USB or SD).

    - Insert memory card into Braille Sense

    - Input command in the boot menu of the DNW window.

      - Input ‘X’ if you use CF memory.

      - Input ‘Y’ if you use USB memory.

      - Input ‘Z’ if you use SD memory.

    - Braille Sense will be reset automatically and upgrade will be processed.

    - Wait until finishing upgrade. Braille Sense will sound “File Manager.”

Many thanks for your effort.

** BY HIMS **