<U2 Firmware Download Manual>

1. What steps you need

- Step1: Download OS Image for fix boot problem. (nk_as.bin)

- If you didn’t install device driver before, you have to install device Driver for USB Download Tool of U2 during process Step 1. Please refer to Appendix A in page number 10.

- Step2: Delete “H432B-Update” folder in flashdisk if it is exist.

- Step3: Download OS Image for normal operation. (nk.bin)

- Step4: Upgrade image with latest version (“H432B-Update” folder)

2. What stuffs you need

- Windows PC (Windows XP or Windows 7)

- USB Cable

- U2ASTOOL.zip

- U2ASTOOL.zip has 4 folders

- “ASImage”: Used for Step 1

- “Dragin”: Used for Step 1

- “H432B-Update”: Used for Step 4

- “RelImage”: Used for Step 3

3. How to follow each step

- Step1: Download OS Image for fix boot problem. (nk_as.nb0)

- Connect U2 and PC with USB cable.

- Unzip U2ASTOOL.zip. You can see four folders in it.

<Picture 1>

- Execute “USBDown.exe” program which is in “Dragin” folder.

<Picture 2>

- Click “OS” button and select “nk_as.bin” file in “ASImage” folder.

<Picture 3>

<Picture 4>

- Check “Auto Detect” check box.

<Picture 5>

- Click “DETECT” button. Button will be changed to gray color.

<Picture 6>

- Press hold “Backward” audio button in front of U2 and press reset button and keep holding “Backward” button about 5 seconds.

- If you didn’t install device driver before, you have to install device Driver for USB Download Tool of U2. Please refer to “Appendix A”.

- If you have install driver, U2 is connected with PC like as <Picture 7>. Green means that U2 is connected.

<Picture 7>

- Click “DOWNLOAD” button. Then downloading will be progressed.

<Picture 8>

- After downloading is finished system will be restarted. It may take about 1 minute.

- U2 will be connected with PC like <Picture 9>. If you can see <Picture 9> then Step1 is finished successfully.

- If you can’t see <Picture 9> after 1 minutes press reset button. Then you can see <Picture 9>

<Picture 9>

- Uncheck “Auto Detect” check box.

<Picture 10>

- Press “Reset All” button.

<Picture 11>

- Step2: Delete “H432B-Update” folder in flashdisk if it is exist.

- When U2 is connected, you can see “H432B-Update” folder in flashdisk like as <Picture 12>. If there is no such folder you can go to “Step3”. If it is exist then delete “H432B-Update” folder.

<Picture 12>

- Step3: Download OS Image for normal operation. (nk.bin)

- Click “OS” button and select image file like <Picture 13>

<Picture 13>

- Check “Auto Detect” check box and click “DETECT” button”

<Picture 14>

- Press reset button and release it. U2 will be connected with PC like <Picture 15>. Green means that U2 is connected.

<Picture 15>

- Click “DOWNLOAD” button. Then downloading is proceeded like <Picture 16>

<Picture 16>

- After downloading is finished system will be restarted. It may take about 1 minute.

- Uncheck “Auto Detect” check box and click “Reset All” button.

<Picture 17>

- Close “USBDown.exe”.

- Step4: Upgrade image with latest version

- After Step 2 is finished U2 is connected with PC as removable disk.

- Copy “H432B-Update” folder to removable disk of U2

- After all files are copied to U2 disconnect USB cable and press reset button.

- U2 will be restarted and upgrade will be started.

<Appendix A>

- Install device driver for USB down program of U2

- When U2 is connected with PC <Picture 18> may be showed up if you did not install device driver for USB download program of U2 before.

<Picture 18>

- Select “No, not this time” and click “Next”.

<Picture 19>

- Select “Install the software automatically” and click “Next”.

<Picture 20>

- Uncheck “Search removable media” and check “Include this location in the search” and select location which you download and unzip U2ADTOOL.zip.

<Picture 21>

- Select “Dragin” folder and click “Ok”.

<Picture 22>

- Installing driver will be progressed.

<Picture 23>

- If <Picture 24> is showed up then select “Continue Anyway”.

<Picture 24>

- After few seconds installing device driver for U2 will be finished.

<Picture25>

*** The End ***